#### Personal Protective Equipment:

Why PPE:

In industrial operations, there are many inherent dangers to life, limb and health of the workers.  The hazard arises out of the use of the dangerous substances, and work in manufacturing process and environments which expose workers to a risk of bodily injury, poisoning or diseases.

The most effective way of controlling a hazard is to eliminate the same by better design, change in process, engineering revision, for guarding by mechanical mean’s.

Inspite of best design and good operating conditions, some situations need the use of personal protective equipment.  Judicious use of personal protective equipment, greatly simplifies the operations to be carried out,  with a reasonable amount of safety.

It must be always remembered that any type of personal protective equipment serves as the thin line of defence.  The personal protective equipment, though, does nothing to reduce, minimize or eliminate the hazard, it definitely set up a barrier against the hazard.  As such, it is very essential that this barrier must be constantly properly maintained.

Many injuries are know to have been avoided by the use of personal protective equipment. The management makes available the necessary personal protective equipment for protection against specific hazard.

It is essential that a right type of personal protective equipment should be used for each job, and this applies particularly to equipment for respiratory protection because equipment of wrong type may prove dangerous to the user.  Any doubt regarding the selection and use of a personal protective equipment should be got clarified in consultation with Safety Officer.

Responsibility :

Plant Managers, Plant Engineers, and Sectional Heads should ensure that required Personal Protective Equipment is available in good condition and provided to employee for use.  Also they should ensure that their Supervisors are thoroughly trained in the use of personal protective equipment.

The employee should realise that personal protective equipments are for their safety and should form the regular habit of using them on all jobs requiring their use.

It is the duty of all personnel working in the plants to know the locations of the personal protective equipments in their plants so that they can be found quickly in emergency or even under black out conditions.  Personal protective equipment must not be removed from their defined locations unless required for use or for stand by purpose as in the maintenance operations.

The record of personal protective equipment issued for the use in plants, and service records of respiratory equipments will be maintained by Safety Section.

All respiratory protective equipments issued for the use or reserve in Safety Section will be inspected and tested periodically by Safety personnel and record maintained.

Classification:

The personal protective equipments which are commonly used in our factory can be classified into two categories as under:

a)  Non Respiratory Protective Equipment :- These types of personal protective equipment are meant for the protection of the various parts of body such as hands, face, eyes, ears, head, torso, etc. against physical injuries.

b)  Respiratory Protective Equipment :- These types of personal protective equipment provide protection to the breathing system of the individuals against toxic and poisonous substances while working in contaminated or oxygen deficient environment.

Non Respiratory protective equipments:

Helmet :- The helmet must be worn at all times in the process areas and when performing mechanical work.  Helmets are very good protection to prevent injuries due to falling objects and striking against hazards.  After long use, the head bands and chin straps may get dirty or damaged and these can be cleaned  or replaced.

2. Ear Plug or Muffler:-  Ear plugs or mufflers are the protective devices for the ears against high intensity noise.

3. Goggles:- To protect the eyes against injuries from flying particles, splashing liquids, and harmful rays that can not be controlled at the source; spectacles, goggles and welding shields are to be used.  (See Section 1.17 on Eye protection).

4. Face Shields:  Certain operations present a hazard to the face as well as to the eyes.  In such cases, face shields provide suitable protection.

5.  Hoods:  These protect the head including face, eyes and neck against specific hazards.   These are made of PVC, neoprene, asbestos cloths, glass fibre, rubber, aluminized asbestos etc. depending upon the type of protection required against.  The hazard could be from excessive heat, high temperature, acids, caustics and other corrosives, solvents, flames,  etc.

Hand gloves:  Protection of hands and arms becomes necessary when handling materials having sharp and rough edges, unwidely objects, hot and molten metals and objects, or toxic corrosive substances.  The kind of glove required will vary according to the possible injury to be prevented viz. cut, puncture, burn, chemical burs, electrical shock, radiation etc.  Several types of hand gloves are used in our factory for different purposes as under:

a) Canvas Handgloves :  For general work and protection against cuts, abrasions, etc.

b) Leather Handgloves:  For handling sharp, rough and heavy objects while doing gas cutting or welding jobs.

c) Fibre glass / Asbestos Handgloves : These ensure good protection against heat flame and against burns and discomfort when hands are exposed to sustained conductive heat.  Asbestos gloves are also useful while handling extremely cold substances such as liquid oxygen, dry ice, etc.

d) P.V.C./Rubber Handgloves: These are made from vinyl chlorides or rubber and  provide good protection while handling acids, alkalies, other corrosive substances and petroleum products.  They are also useful while handling extremely cold chemicals such as liquid ammonia.

e) Rubber Handgloves (Electricians):  These are tested to specific voltage and are useful for protection against electric shock.

f) Caution: -

i)  Handgloves must be regularly inspected for normal wear and tear, especially the P.V.C. and Electricians’ Rubber handgloves.  Any leak in such gloves will render the same useless.  A daily visual inspection and air test by mouth before use are recommended for P.V.C. and Rubber handgloves.

ii)  Handgloves must never be worn while working on or near revolving machineries such as drills, lathes, grinders, etc. with which the handgloves may be caught in moving parts.

Aprons: Aprons provide protection against specific hazards to the abdomen and chest.  The aprons are made of different materials for various use as under:

a)  Bluedrill:  For protection against flying particles while working on machine tools.

b)  Leather:  For protection against hot sparks, bits of molten metal and heat while doing welding and gas cutting jobs.

c)  Asbestos:  For protection against sustained conductive heat, and sparks.

d)  P.V.C./Rubbers:  For protection against corrosive substances, and splash and spillage of corrosive chemicals and liquids.

Suits:  Suits provide protection to complete body from head to toe when worn along with gloves, hood and shoes.  The suit is either a combination overall type or consists of a pant and coat.  The suits available in our factory are made of different materials as under:

a) P.V.C.:  For protection against corrosive and harmful substances such as Nitric Acid, Sulphuric Acid, Phosphoric Acid, Caustics, Liquid Ammonia, Methanol, etc.  The suit must be worn along with food and gumboots.  In addition to the complete suit, long coats made of PVC are also available.

b)  Asbestos / Fibre Glass:  For protection against heat, high temperature, sustained conductive and radiant heat.  The hood and shoes made of the same material are part of the suits.

Safety Belts: Safety belts with life lines are worn by the people who work at heights or enter and work inside confined spaces, both above ground level and below ground level.  While working at heights the life line must be securely tied to a permanent structure whereas inside a confined space, the same should be held by a person standing outside.  It must be ensured that harness, belt, buckles & D-ring/ring and rope are in good condition.

Respiratory protective equipment:

It is of utmost importance that a proper type of respiratory equipment must be selected or else result may be suicidal.  The following are main factors to be considered in the selection of the proper respiratory protective equipment for any given situation.

a)  The nature of hazardous operation or process.

b)  The type of air contaminant including its physical properties, chemical properties, physiological effects on the body, and its concentration.

c)  The duration for which respiratory protection must be provided.

e)  The state of health of the person involved.

f)  The functional and physical characteristics of respiratory protective equipment.

The employees must be trained in proper selection and use of respiratory protective equipment.  They must also develop a habit of upkeep, maintenance and proper storage of respiratory protective equipments.

The following protective equipments of various types and uses are available in our plants for respiratory protection against harmful and toxic environment.

Gas Mask Canister Type:

a)  It consists of a face piece connected by a corrugated breathing tube to a canister.  The face piece is provided with a visor, head harness and exhalation and inhalation valve. The canister is held in a harness for suspending in the neck and tieing in the waist.

b)  The canister contains certain chemicals that purify, absorb or neutralise the contaminants and allow only fresh air to pass through to the face piece.

c)  Because no single chemical has been found to remove all gaseous contaminants, the canister must be chosen to suit the specific need. However, there are canisters available for protection against more than one contaminant.

d)  Canister type gas masks are for emergency protection in contaminated  environment. They do not provide protection in oxygen deficient atmosphere.  Their effectiveness is limited to use in atmosphere containing at least 16% by volume of oxygen and not more than 2% by volume of those gases for which it is designed except for ammonia, for which the limit is 3%.

e)  The period of protection that a canister gas mask will provide depends upon following factors:

- Type and size of the canister

ii)   Concentration of the gas or vapour.

iii)  Activity, health and breathing efforts of user.

iv)  Duration of use at each time.

f)  By keeping proper records of time of canister’s use and percentage of toxic concentration to which it was exposed, the approximate balance life of the canister can be ascertained.  The life so determined, can not be relied upon fully  because of several factors controlling each as enumerated above, besides its manufacturing date and mode of storage.

g)  When the face piece is properly adjusted air tight and all the parts of gasmasks are free from any cut, puncture, and mechanical defects, leakage of harmful and toxic gases indicated by odour, test and / or irritation of eyes, nose and throat is evidence that the canister is exhausted.  However, in case of canister gas mask meant for odourless and tasteless gases such as carbon monoxide, a window indicator with reference & indicator sections is provided on the canister.  In such a case when a colour of indicator section approaches to the colour of reference section, the canister is exhausted.

h)  When it is detected that canister is exhausted, a canister must be changed with new one in fresh air.

i)  The canister type gas mask must never be used in oxygen deficient atmosphere and inside confined spaces.

Canister Types : The following types of canisters for protection against various harmful and toxic gases are available in our plants for use.

Type and Protection against:-

Ammonia Canister – 3% by volume of ammonia.

Chlorine Canister – 2% by volume of chlorine.

All service (Permissible ‘N’ type Canister): --3% by volume of ammonia or 2% by volume of total poisonous gas or individual gas i.e. Acid gases, organic vapours and carbon monoxide, H2 S etc.

How to Use gas mask:

a)	 Inspect the gas mask for any defect and select the right type.

b) 	Remove the bottle seal of the canister and put on the canister harness  and face piece.

c) 	Adjust the head harness until the face piece fits closely, comfortably and air-tight.

d) 	Ensure that there are no kinks in the breathing tube.

e)	Grasp the breathing tube tightly and close the air intake.  Breath deeply until the face piece collapses on the face.  If correctly adjusted the face piece will remain collapsed until the air intake is opened.

f) 	Breath naturally and observe the resistance to breathing and check for the correct working of the canister.

Air-Hose Mask with Blower:

a) 	 It consists of a hand operated centrifugal blower mounted in a box, to face pieces with breathing tubes, safety belts with life line, check valve mounted on rear side of safety belt and reinforced rubber hoses.

b)  	This respiratory protective device supplies air from an uncontaminated source when the blower is operated.  The air from the blower passes thorough the reinforced rubber hose to the face piece worn by the user.

c)  	The air is drawn through the check valve and breathing tube for inhalation and exhaled air goes out through exhalation valves, which is not thus rebreathed.

d)  	In case the blower is stopped accidentally, the wearer can escape by his own efforts of breathing out of the remaining air in long hose.

e)  	The Air Blower Hose Mask can be used by persons working in contaminated atmosphere irrespective of concentration of harmful and toxic gases outside or inside a confined space and even in places where oxygen deficiency exists.

f)  	This equipment can provide protection to two men simultaneously

with freedom to move in 150 ft. radius. If only one person uses this equipment, the hose length can be extended to 300 feet.

How to use HOSE MASK with Blower:

a)	Keep the blower in fresh air, away from the contaminated environment.

b)	Inspect the face pieces, rubber hoses and blower to make sure that they

are in good condition.

c)	Lay the reinforced rubber hoses on the ground and connect the couplings.

d)	Put on the Safety belt with sufficient length of line.

e)	Connect the reinforced rubber hose to the outlet of the blower.

f)	Operate the blower and make sure that air is coming out and check for

leaks in hose and coupling joints.

g)	Connect the other end of the reinforced rubber hose to the check valve

adapter mounted on the rear side of safety belt worn by the user.

h)	Put on the face piece to fit closely, comfortably and airtight.

i)	Connect the breathing tube coupling to the outlet of the check valve

provided on the rear of safety belt worn by the user.

The blower equipment should be positioned in fresh air only.  Before

proceeding to the contaminated atmosphere, check the connection for air

tightness and ensure that there are no kinks in the breathing tube and the

reinforced rubber hoses.

Self contained compressed air breathing apparatus:

a) 	The apparatus consists of a high-pressure air cylinder, cylinder valve,

demand regulator and a  face piece with breathing tube and exhalation

valve. The cylinder is mounted on back plate having harness for shoulder,

chest and waist.

b)  	The apparatus can be used in atmosphere that are immediately hazardous to  life.  It affords completely respiratory protection in any toxic, harmful or  oxygen deficient atmosphere, regardless of the concentration of the contaminants.

c)  	It allows complete freedom of movement with limiting factor of time which is 30 minutes in respect of the apparatus available in our factory.

d)  	The air is drawn from the cylinder through a high pressure hose to the demand regulator and passed over the face piece through corrugated breathing tube for inhalation.  The exhaled air goes out through exhalation valve provided in face piece and thus is not rebreathed.

e) 	The demand regulator is provided with two knobs, one yellow and the other red.  The yellow knob is the main valve control and used in normal working condition.  The red knob is the bypass valve control and is used in case the yellow knob fails or demand regulator is damaged.  The duration of air supply is using the red knob is greatly reduced and as such this operation more should be used to escape from the contaminated area not to continue work in case of failure of demand regulator and yellow knob.

f) 	The air cylinder, when fully charged, has a pressure of approximately 2000 p.s.i.g. and will last for 30 minutes.  When the cylinder pressure, during the use of apparatus, drops to about 4000 p.s.i.g. a warning device in the apparatus gives an alarm which continues till the cylinder becomes empty.  The approximate time of alarm is 5 minutes.

g)  	During the use of apparatus when the alarm is sounded, a user must come out to the fresh air and change the cylinder with a full one.

How to use Breathing Apparatus :

The putting on of self contained compressed air breathing apparatus is one man operation and needs regular practice and training for immediate use in emergency.  The apparatus can be put on in the following manner:

a) 	Take out the apparatus from the case by holding the shoulder straps.

b) 	 Swing the cylinder on the back and insert both hands under the shoulder belt.

c)  	Adjust the shoulder straps by pulling downwards backside with both hands.  If necessary, give a jerk to the cylinder resting on back for better and quicker tightening of the shoulder strap.  Adjust the waist belt:

d)  	Put on the face piece and adjust the head harness until the face piece fits closely, comfortably and air tight.

e) 	Grasp the breathing tube tightly and close the air intake.   Breathe, deeply until the face piece on face.  If correctly adjusted, the face piece will remain collapsed until the air intake is opened.

f)	Ensure that red knob on demand regulator is fully closed.

g) 	 Release the cylinder valve safety lock and open the cylinder valve by turning the wheel two to three full turns.  Check the pressure gauge for full indication.

h)  	Open fully the yellow knob on demand regulator.

i)	 Connect the breathing tube coupling to the outlet of the demand regulator before entering in the contaminated area.

Changing Air Cylinders:

a) 	 Uncouple the breathing tube from the demand regulator.

b) 	 Release cylinder valve safety lock and close the cylinder valve fully.

c)  	Open the red knob on demand regular to depressurise the high pressure hose.

d) 	 Uncouple the high pressure hose from the cylinder valve.

e)  	Pull the cylinder clamping lever to release the cylinder.

f)  	Remove empty cylinder from the back plate and replace with full one.

g)  	Replacing the charged cylinder can be done in reverse order of the above steps.

Maintenance:

a)  	All the respiratory protective equipments must be maintained in good working condition.  When any part shows signs of failure, it should be replaced with proper part.

b)  	The face piece and breathing tube which form the part of the respiratory protective equipment  must be washed, cleaned and sterilized  at regular intervals and after each use.

c)  	All the respiratory protective equipments, when not in use, shall be kept in the carrying cases and stored in  a cool, dry and ventilated place.

Dust Respirator:

a)   	Dust respirator is simple device for protecting the respiratory track and lungs from dust of material such as KCI. D.A.P., Rock phosphate, Sulphur and fertilizers.  It consists of a mouth piece, filter, exhalation valve and head band.

b) 	 While breathing, the dust particles in the atmosphere are filtered, collect on the filter and only free air is allowed to be inhaled.  The exhaled air goes out through exhalation valve and thus is not rebreathed.

How to use Dust Respirator:

a) 	Check the dust respirator for clear filter and correct working of exhalation valve.

b)  	Put on the mouth piece avoiding any gap between mouth piece and face and adjust the harness.

c)  	Choked filters should be replaced with new ones as and when required.

# Helmet

The helmet must be worn at all times in

the process areas and when performing

mechanical work.  Helmets are very good

protection to prevent injuries due to

falling objects and striking against

hazards.  After long use, the head bands

and chin straps may get dirty or

damaged and these can be cleaned  or

replaced.