SAFETY  AND HAZARD MANAGEMENT IN FERTILIZERS INDUSTRY

A.S. Karmarkar

- Preamble

“Accidents” & “Disaster” can strike at any time ! Though the statement is universal truth, Man made disasters or accident really do not strike suddenly.  It is an aggravated incidence due to human failures.  Natural disasters like earthquake, storms, floods can strike without warning.  However, if we are prepared, mitigation and rehabilitation measures can be made very effective so that minimum loss to the property & human life do occur.

Histories of Major industrial disasters like “Bhopal Gas leakage” or Natural disasters like recent “Tsunami” are eye openers.

Entire mankind has emerged as ruler of the planet “Earth”, only by virtue of learning through experience.

Recent technology developments have provided man “Extra senses” and “Third eye”.  However to forget & err is a human.

To eliminate, minimize & control industrial Incidence / accidents / Disaster a “sound” safety management system is an essential requirement.

In this article, I have covered a brief overview of “Safety & Health Management System” implemented at M/s. Rashtriya Chemicals & Fertilizers Ltd, Trombay unit, Chembur, Mumbai.

- Company Profile in Brief

RCF was part of erstwhile FCI.  The company (as FCI Trombay) unit was established in 1960 and Manufacturing started in the year 1964 with Ammonia, Urea, Suphala (NPK), Nitric Acid & Sulphuric Acid.  Gradually other products were also introduced such as Methanol, Methylamines, Conc. Nitric Acid, Ammonium bicarbonate, Phosphoric Acid, Sodium Nitrate.

In the year 1978, upon reorganization of all fertilizer companies Trombay unit was made a separate company named as M/s. Rashtriya Chemicals and Fertilizers Ltd.

Our company further expanded in the year 1981 by setting huge Urea, Ammonia Manufacturing plants at Thal, 100 km. away from Mumbai.  Thal unit has also further diversified in to various industrial chemicals as Formic Acid, Dimethyl Formamide etc.

To further strengthen the commitment, RCF has established Safety Management System and Environment Management System, which got duly certified to meet OHSAS 18001 and ISO 14001 – 2004 compliances.

Operation of such huge fertilizer & chemical plants involves handling of large quantities of flammable materials like Natural Gas / Naphtha & explosive and toxic gas such as Hydrogen, Carbon Monoxide etc.  While operating such plants, processes are being carried out at temperatures ranging from – 190o C to as high as 1000oC, so also the pressure ranges from vacuum to as high as 200 bar, thus posing various safety, health & environment hazards.  It is therefore imperative that it is impossible to operate, handle, and transport such hazardous chemicals without attaching top most priority to “Safety”, “Health”, & “Environment”

All plants at our Trombay Unit have been up graded to the current state of art technologies with precise, accurate process controls through DCS system.  Right from the inception of this unit, we have had full-fledged “Environment cell” and Safety & Fire department with exclusive professional staff to manage these departments.  Top management of RCF has always been committed for continually improving safety, health and environment performance.

- Philosophy of RCF and Safety Policy

Industrial Safety is part and parcel of RCF ‘s corporate mission.  The following declarations in the company’s mission very explicitly reveal the positive intention of the company towards Occupational Health & Safety.

The company is accountable to obtain for its employees as decent a standard of living and as good a quality of life as possible, in line with general socio-economic conditions in the country.

The company is accountable to improve environment and to minimize the harmful emissions, atmospheric discharges and effluents to the maximum extent technologically possible.

The company is always on the lookout of new technologies for environment protection and adopts the same whenever and whichever is available.

The company has translated the mission into its Occupational Health & Safety policy with specific emphasis on safety and in detailed policy statements.

- Occupational Health & Safety Policy

- We at Rashtriya Chemicals & Fertilizers Limited, a company engaged in manufacture of Chemicals & Fertilizers, commit to maintain a safe and healthy work environment in compliance with all relevant regulatory obligations.

- We shall strive to continually improve our Occupational Health & Safety Management system for prevention of accidents and reduction of significant health risks to the employees, contract labours and others due to their presence within the factory.

- We endeavor to achieve this through:

- Minimizing any adverse impact of our operations, activities and products on the Occupational Health & Safety

- Involvement of employees in identification and control of Occupational Health & Safety issues of the site.

- Training of our employees, contractors & other agencies to promote safe work culture at our site.

- Periodic analysis of information on Occupational Health monitoring, incidents and accidents to improve our processes, practices and facilities.

- Safety Department – Role & Responsibility

Safety Department, with its professional safety personnel, is the prime mover to establish strong safety management system in the organization.  However, one must not forget that “Safety is line Management responsibility and staff function”

Safety Officers assist the line Management to ensure safety standard, work practices, statutory compliance in all operational & maintenance activities.

Safety Department also gives constant feed back to the top management on various safety related issues for continual improvement.

- Safety Management Network Adopted at RCF,Trombay

Top Management

CMD/ Directors

Strong intent ,  commitment & control

Various legislation

Safety, Health, Environment & Quality Policy

Line Management Responsibility to execute policies

Development of sub policies & rules, standards, procedures

THINKING AREA

FEEDBACK & APPRAISAL

ASSISTANCE

PERIODIC REVISION

#### SAFETY DEPARTMENT

COMPLIANCE

PREPARATION & DEVELOPMENT

Investigating & reporting, remediall measures

Inspections & corrective actions

Identification & removal

##### WORKING AREA

Unsafe Actions

Unsafe Conditions

## Accidents

Incidents

## Hazards

- Allocation & delegation of responsibilities for plant safety

- Top Management:

a) 	Chief general Manager [Occupier] :

Is responsible for framing a comprehensive Occupational Safety, Health and Environment Policy and integrating this policy with the Company’s objectives and getting the same approved by CMD & Board of Directors for adoption by the Unit.

b)	General Manager [Factory Manager]:

Shall direct implementation of Occupational Safety, Health and Environment Policy and review recommendations from DGMs for approval as and when required and review the Safety performance periodically.

c)	Dy. General Managers/ Operations Manager / Chief Engineers:

Shall head the Safety Committee and shall take decisions within the framework of Occupational Safety, Health and Environment Policy for effective implementation. They shall review the safety performance periodically.

- Line Management:

The Managers/Engineers and Sectional Heads of plant have the definite responsibility of ensuring the following,

a) Safety in working places

b) Necessary protective equipment and its use.

c) Properly maintained tools and equipment.

d) Properly planned work performed in a safe manner.

e) Observance of Safety Work Permit procedure and such other safety procedures of work.

f) Observance of general and special safety instructions by the Workmen.

g) Assignment of employees to jobs which they are capable of doing safely.

h) To take prompt measures  (including stoppage of process if necessary) to control any emergency situation which is dangerous and threatening to life, property and/or environment

- Safety Officer:

a) Safety Officers are appointed to ensure proper observation of Safety rules, Safety standards and Statutory Safety Provisions with respect to Occupational Safety, Health and Environment.

b) Safety Officer shall monitor Occupational Safety, Health and Environmental activities at all levels. They will function as expert and assist to implement Occupational Safety, Health and Environment Policy.

c) Safety Officer will monitor the line management, by constant vigilance and inspection, in the enforcement of safety orders and other statutory Occupational  Safety, Health and Environmental provisions . Safety Officers are empowered to check registers, log books and any other document necessary and also to record statements.

d) Safety Officer will disseminate all relevant information to employees, community, mutual aid members and regulatory bodies within the scope/ framework of company’s business policy and relevant statutory provisions.

e) Safety Officer will motivate employees towards safe behavior by observing National Safety Week Celebration through various Safety Competitions  such as posters, cartoons,  essay,  elocution,skit and poetry contests.

f) Safety Officer will prepare reports on Occupational Safety, Health and Environmental surveys, inspections, audits, risk assessments, emergency plans, mock drills and implementation of recommendations made in the report.

g) Safety Officer will co-ordinate with HRD to organize regular workshops/ seminars for safety awareness and also communicate with all employees through safety news, bulletins and circulars.

h) Safety Officer will prepare necessary reports after investigation of accidents and advice corrective action to be taken to eliminate similar accidents in future.

4. Employee:

a)	Everyone is responsible to act in a manner so as to provide,

i)	Safety to himself.

ii)	Safety to his fellow employees

iii)	Protection to the public

- Protection to the company’s property

b)	Every employee is expected to study the Safety Rules and apply them to his work.

c)	Before attempting any work under conditions that one considers to be unsafe, the same shall be brought to the notice of the Supervisor-in-charge of the work and his advice sought.

d)	No employee in the factory will misuse or neglect to make use of any appliance provided in the factory for the purpose of securing the health, safety and welfare of employees.

e)	No employee will willfully and without reasonable cause, do any thing, which is likely to endanger himself or others.

f)	Every employee if they have reasonable apprehension that there is likelihood of imminent danger to their lives or health due to any hazardous condition he should bring the same to notice of his supervisor. He may also bring the same to  the notice of his Head of the department and Safety Officer.

- Accident Prevention Strategies

As per latest definition of Accident / Incident “Accident is a result of contact with source of free energy beyond threshold limit value”.

It is very easy to understand & predict where contact with source of energy is likely to come in a way.  The contact can be nullified by proper job preparation, safe work practices, isolation, use of personal protective equipments, etc. However, Natural source of energy / forces such as gravity & climatic conditions can not be removed. Utmost care for removal of unsafe condition & unsafe action is required.

Involvement of people at all levels- Though the “ Safety’ is every one’s responsibility, to inculcate Safe work culture among all levels of employees, simple philosophy of “No work would be said rightly done unless it is safely done” is conceived.

- Plant Level Safety Committee

Plant level safety Committee is constituted in each plant and bimonthly meetings are conducted to discuss all safety related issues. Safety Officer attends the meeting and provide all assistance required.

- Central Safety Committee

Safety and environment committee comprising of Management and trade union representatives discusses the matter related to Safety, Health and Environment and gives recommendations / suggestions to General Manager for implementations.

The objectives of these committees are :

1] To review accident statistics and assess the status of remedial measures suggested

preventing accident.

2] To inspect plants and suggest corrective action to improve working conditions and

environment.

3] To pursue and follow up with the plant incharges for implementation of safety

measures.

4] To suggest and pursue with the management a program for better health and

hygiene.

5] To suggest motivational programmes for safety and health and assist in organizing

such programmes.

Committee meeting is conducted quarterly and review/minutes are widely circulated.

- Monthly Safety Review Meeting by Occupier

Monthly Safety review Meeting is conducted by the Occupier of the factory. All safety related issues, accidents, incidences, unsafe work practices are discussed in details. Specific responsibilities are fixed with target dates.

- Best Practices Observed to Avoid /Eliminate Accidents / Incidence

- Incident register – Open register is maintained in each plant where employees can record unsafe acts, unsafe conditions, untoward incidences, near miss accident.  This register is reviewed by each Operation Manager on day to day basis and immediate corrective action is taken.

- Surveillance Inspections -  Plant surveillance inspection is conducted in each fortnight by a Surveillance Committee consisting of Plant officials and employees and Safety representative.  Report generated is submitted to operation manager for corrective action.

- Day to day report by Safety Officer  - Unsafe actions, conditions observed during day to day inspection rounds are brought to the notice of occupier and are discussed on each day in morning production review meeting.

- “Suraksha Shree” Award – One employee from each plant is identified as “Suraksha Shree” employee. Selection of such employee is based on his safety performance review, awareness, alertness and follow of safety precautions.  Such employee is awarded with a “certificate” and a badge.  He is empowered to work as “Plant Safety Observer”

- Good House Keeping contest – To motivate plant employees for good housekeeping A contest is organized every year.  Plants are inspected in each quarter by a “Good House Keeping” committee. Based on the performance of each plant, as per check list, marks are awarded.  Winner plant is awarded with good house keeping flag and annual winner is awarded with “Good Housekeeping Trophy”.

- “Best Safety Performance Award” – This is a Corporate award for one Plant in each of the unit to recognize their annual Safety performance and achievements. Scheme is implemented in the form of contest among Plant.  Appointed committee inspects the Plant.  All aspects of “Safety” are inspected as per check list.  More emphasis is given on corrective actions taken in follow up inspection round.  Winner plant is awarded with " Late Shree Duleep Singh Trophy” for best safety performance during the year and cash award of Rs. 25,000/-

- Safety of the Contractor employees -  All contractor employees are covered for  “Safety Training “ before allocation of work in plant.  On the job safety training is given at regular interval.  Class room training is given periodically.  All contractor employees are subjected to periodic medical examination to assess health status. Also it is ensured that adequate personal protective equipments are provided to the contractor employees and its use is monitored on regular basis.

- Equipment inspection / Maintenance strategy

- a) Maintenance and Inspection System

To eliminate / minimize the hazard, top most maintenance strategy is always necessary. In RCF Ltd., top most priority is given for maintenance. Maintenance schedule in brief is as below,

1. Annual overhauling/scheduled maintenance

2. Predictive maintenance

3. On line maintenance

- Annual overhauling/scheduled maintenance:

This is carried out for the entire RCF complex at the beginning of every new financial year i.e. in April, so that all plants can run efficiently and smoothly without any interruption for the remaining period of the year so as to achieve production targets. Annual shutdown jobs are also planned on the basis of periodic monitoring of the machines where on line repair is not possible.

Under this program each critical machine is shutdown and partially/completely dismantled for thorough inspection of worn out parts.  Complete overhauling of critical equipments is done.  This Programme of periodic dismantling and inspection has distinct advantage of decreasing the frequency of breakdown repairs.

During the annual overhaul, periodic NDT of all static equipments is also carried out thoroughly. NDT work covers, thickness testing, hydraulic testing of pressure vessels periodically and all other techniques if necessary such as surface crack detection, radiography, ultrasonic flaw detection etc..

- Predictive maintenance (on line advance warning of future maintenance problems):

In RCF a special cell called Condition Monitoring Cell is set up to take care of all critical rotating equipments in the factory. A plant wise list of all critical equipments is made and schedule of periodically (once in 15 days) monitoring of these equipments is strictly followed.

Condition of these equipments is monitored with the help of vibration measurements and spectrum analysis so that any abnormality that develops is repaired.

In order to avoid  further damage to machine if required, planned shutdown is taken with minimum of downtime and expenditure . A history of all critical equipments is maintained for future reference.

- On line maintenance:

On line sealing techniques are used to stop the leakage’s on main lines/equipments which otherwise require shutdown of the plant and thus plant is kept running continuously and safely.

In addition to above methods used, the following also forms part of maintenance system being followed,

a)	Daily checking /check list being carried out in each plant for all critical equipments and corrective action is taken accordingly.

b) 	Day to day preventive maintenance being carried out on standby equipments so that they remain available all the time.

c) 	All pressure vessels being periodically( once in four years) hydraulically tested.

d)	NDT being carried out periodically for all static equipments.

e)   Structural stability of plants and buildings being inspected and studied periodically through a separate competent authority and remedial measures suggested by them are implemented.

- Painting with suitable paints is carried out periodically for complete structure and equipments of RCF complex  to take care of atmospheric corrosion.

- Safety Inspections

The following safety inspections are carried out by Safety Department ,

i) Inspections of unsafe conditions and unsafe actions on daily basis.  .

ii) The condition of safety equipments, fire extinguishers and fire hydrants is checked every month. Work environment monitoring and Noise level measurement is carried out twice a month.

iii) Surprise inspections are carried out to check and correct violation of safety precautions suggested for jobs under Safety work permits.

iv) Checklist based inspections of material handling equipments, pipe racks, machine guards, storage tanks and Illumination survey are carried out at six months frequency.

v) End-to-end survey by Safety Committee and Safety Audit Committee is carried out to cover all the plants in period of 1 year.

- Trouble Shooting

Trouble shooting is carried out by Technical Service Team, In trouble shooting job the typical and chronic problems in process, failure of equipments etc.. are studied thoroughly to establish the reason of failure and remedial measures are suggested to avert such problems or failures in order to improve the performance and safe running of the plants.

- Emergency Preparedness

- Risk analysis of all plants are being updated and evaluated. Recommendations suggested are implemented with special emphasis on elimination of major accident hazards and updating of emergency control plan.

- Each location/ plant has been provided with adequate personal protective equipment, safety and fire fighting equipment and first aid/medical facilities.

- Each plant in consultation of safety and fire department is conducting mock drills  (emergency exercise) periodically to ensure effective co-ordination during emergencies.

- Material Safety Data Sheets of hazardous chemicals used in Trombay unit have been made available to the employees, transporters and consumers. The MSDS is revised in case of addition of new chemicals.

- On-site Emergency Control Plan has been made available indicating hazards  and  probable  MCA  scenarios  showing   the	responsibility of   each   employee  in   case of any emergency.  Emergency Control Plan is updated based on experience of emergency exercise and any amendment in statutory regulations.

RCF Ltd. is also an active member of Mutual Aid Response Group, Chembur-Trombay Region and also has entered into a written agreement with nearby refineries for Emergency Assistance.

- Promotional activities

a)  Celebration of Safety Day / Safety week campaign

Safety day is celebrated on 4th March every year on a grand scale followed by  observance of safety week.  Various contests are organized for the employees and winners are awarded with cash prizes.  Various programmes such as display of posters & banners, safety exhibition, talk by eminent faculty, screening of films etc are organized during the week.

- Celebration of Fire service week

Fire service day is celebrated on 14th April followed by fire service week. Various contests for employees are organized during the week.  Special efforts are made to create awareness among employees by “Training Programmes”, “Mock Drills”, and Demonstrations etc during the week.

- Safety Information Bulletin

Safety information bulletin is published by safety department every month & is widely circulated. Current safety & Health related a topic is kept as a focus of each such bulletin.

- Effective use of Intranet

To propagate information on Safety, related articles, safety manual, information sheets,  material safety data sheets are made available on “Safety” web site.   Photo gallery is also made available in the web site where photographs of “events” “Occasions”, “Celebrations”, “Unsafe conditions”, “Unsafe actions” are displayed for viewing by employees.  To facilitated the photo coverage safety department is provided with “Digital Camera”

- Occupational Health & Hygiene

- To ensure proper health of the employees six monthly medical check up is carried out for all employees.  Employees are advised on health related matters on regular basis.  Special  drives such as mass vaccination programmes, Lectures by eminent faculty

are organized periodically. Training on basic first aid & occupational health management is organized regularly to cover all levels of employees.

- Hygiene is constantly monitored by Occupational Health doctors and welfare officers, in all plants, offices canteen etc.

- Sanitation facility is inspected on regular basis.

- Welfare blocks are inspected regularly to ensure sufficiency of facilities such as water coolers, wash basins, lockers, chairs and tables etc.

- Community awareness

Extra efforts are made for the community awareness by

- Organizing factory visits.

- Distribution of information pamphlets

- Organizing training programmes.

- Top most priority is given “Round the Clock” to attend to any public complaint.

- Conclusion

Safety is round the clock business and effective Safety and Hazard Management is only possible by constant vigil.  Safety requirement is not a ritual and needs sustained efforts without any relaxation.  After all, “Accident Prevention” is benefaction, where as accident compensation is an apology.