STORAGE & HANDLING OF FLAMMABLE, TOXIC, POISONOUS AND CORROSIVE MATERIALS

GENERAL.

The storage and dispensing of flammable, toxic, poisonous, corrosive and explosive materials constitute both a fire hazard and health hazard.  The following information, precautions and rules regarding such materials should be carefully understood and strictly observed.

Flammable materials.

Practically all combustion’s take place between oxygen and fuel in its vapour or some other finally divided form.  For a solid to burn, it is necessary for a portion of it to be heated up and evolve vapours to catch fire.  Since at a given temperature liquids are generally at a higher vapour pressure than solids, flammable liquids are easily ignitable,. Flammable dusts are equally easy to ignite as vapours or gases.  Fires of solids propagate slowly, fires of liquids propagate relatively rapidly and dust, vapour or gas fires propagate so rapidly that they often seem to explode.

OXIDISING AGENTS.

- Oxidising agents are sources of oxygen, which is one of the necessary components of a fire. Normal air containing about 21% oxygen is the primary source. However, many other materials are so constituted chemically that they can supply oxygen to a reaction, even in the absence of air. Some of these oxygen suppliers require heat before they will yield oxygen; others evolve significant amounts of it at room temperature. The following classes of compounds are noted for their ability to supply oxygen.

- Organic and inorganic peroxides, oxides, permanganates, chlorates, perchlorates, persulfates, organic parselenatesm perbromates, chromates, dichromate’s, ozone and perborates.

- NPK fertilizers and Sodium Nitrate/Nitrite are the substances, which are potential oxidising agent. Any fire in these products can result in serious consequences, especially in storage silos, sheds and bunkers, etc.

Water sensitive Fire and Explosion Hazards.

These are materials, which react with water, steam or water solutions to evolve heat, flammable gases or explosive gases.  Examples of such materials are Lithium, Sodium, Potassium, Calcium, Rubidium, Cesium, alloys and carbides of the above. Hydrides, nitrides, sulfides, carbides, borides, silicates, telluride’s, selenides, arsenides, phosphides, and anydrides and concentrated acid or alkalis.

Of the above list, the first eight are materials which react exothermally with moisture and evolve hydrogen gas, the next nine react rapidly with moisture to evolve volatile, flammable, sometimes spontaneously flammable and/or explosive hydrides acid anhydrides and concentrated acid or alkalise react with moisture to evolve heat.

In our factory, Sulphuric acid, nitric acid (63% & 100% concentrations) and strong phosphoric acid products and caustic soda, caustic potash and lime are the examples of water sensitive materials.

Fire & explosion hazards of acid and acid-fume sensitive materials.

These are materials, which react with acid, and acid fumes to evolve heat, hydrogen and inflammable and/or explosive gases.

Examples are – lithium, sodium, potassium, calcium, rubidlum, ceslum, alloys and amalgams of the above, hydrides, nitrides, sulfides, carbides, borides, sulicides, telluride’s, serenades, arsenides, phosphates’, concentrated alkalis, metals arsenide’s, selenium, tellurium and cyanides.

Toxic materials

- These are materials, which under either normal conditions or disaster conditions, or both, can be dangerous to living things around them. For example, carbon tetrachloride stored in a poor ventilated place under normal conditions can evolve enough vapour to render the storage area toxic. Under disaster conditions of high temperature, if carbon tetrachloride decomposes it can form significant quantities of the highly toxic phosgene.

- The examples of toxic materials in our factory are Ammonia, carbon monoxide, hydrogen Sulphide, oxides of nitrogen, Sulphur dioxide, Sulphur trioxide, Methanol, Amines, Dimethylether, Trichloroethylene etc.

Corrosive Materials

Corrosive materials include acids, acid anhydrides and alkalise. Such materials often destroy their containers and get into the atmosphere of a storage area. Acid fumes react to evolve toxic fumes with sulfides, Sulfites, cyanides, aresenides, tellurides, phosphides, borides, silicides, arbides, fluorides, selenides. They liberate hydrogen upon contact with meals and hydrides. Alkalies may liberate hydrogen upon contact with Aluminium etc. Acid mists or fumes corrode structural materials and equipments and are toxic to personnel. Strong acids and alkalies will cause serious burns and eye damage.

STORAGE

General

- Bulk stock of all categories of the above mentioned materials should be stored in approved type of building specially allotted for this purpose.  Only the minimum essential quantity of the above materials required for the efficient operation and maintenance jobs, should be kept in a separate sub-stores under lock.

- All stores and sub-stores should be kept well ventilated to prevent the accumulation of explosive and toxic gases.  Regular gas tests should be conducted to ensure that the ventilation is adequate.

- Smoking or use of naked lights in or near store building is strictly forbidden.

- Name of the materials stored should be displayed near their respective storage for identification.

- Suitable Fire Extinguisher should be kept in readiness in the area and “No Smoking” signboards should be displayed.

Storage of Flammable Materials

- Highly flammable materials must be kept apart from powerful oxidizing agents, materials which are susceptible to spontaneous heating, explosives, and the materials which react with air or moisture to evolve heat.

- The storage of flammable materials must be electrically grounded and periodically inspected for grounding.  Bare filament heaters or other sources of ignition must be kept away.

Storage of Oxidising agents

- Oxidizing agents should be stored separately.

- Oxidizing agents should not be stored close to flammable materials, especially close to liquids of low flash point.

- The storage area should be kept cool and ventilated and should be fire proof.

- Normal fire fighting equipments having blanketing or smothering effects are less effective because oxidizers supply their own oxygen.   As such, it is best to keep the fuel away.

Storage of Water Sensitive Material.

- Such materials must be stored in well-ventilated, cool, dry areas.

- No automatic sprinkler system should be used in such storage areas.

- In no case water should be allowed to enter such storage areas.

- The building should be waterproof, located on high ground and separated from other storage.

Storage of Toxic Materials

Materials which are toxic or which can decompose into toxic components due to contact with heat, acid or acid fumes should be stored in a cool, well ventilated place, out of the direct rays of the sun, away from the areas of high fire hazard and should be periodically inspected and monitored.

Transferring or Dispensing Flammable Liquids

Pneumatic transfer of flammable solvents and liquids should be affected only by inert gas.

Flammable liquids shall only be introduced into vessels through inlet pipes connected with the bottom or the side of the vessel and properly bonded and earthed.

Effective steps should be taken to prevent leakage of such Liquids into basement, pumps, trenches or drains and confine any escaping liquid within safe limits and also to avoid formation of explosive mixtures of vapour and air, especially during transfer.

Accidental mixture of flammable liquids should be prevented.  Gasoline mixed with fuel oil may change the flash point sufficiently to make the fuel oil hazardous in ordinary use.

Different equipment for transferring different flammable liquids should be used.

Flammable liquids must not be kept in open or in badly sealed containers.

The use of flammable liquids such as petrol, or naphtha for cleaning purpose is forbidden.

All flammable materials such as petrol, kerosene, paints, varnished, solvents, lubricants both liquid and solid should be dispensed only in approved containers designated for each item.

Special care should be taken to avoid contamination of lubricants during handling and dispensing.  Measuring vessels and funnels for dispensing lubricants should be kept in dust proof cupboards and cleaned before use with fluff less wipers.  The use of cotton waste for cleaning such equipment is forbidden.

Handling of Toxic and Corrosive materials:

When loading, transporting or unloading toxic and corrosive materials, HANDLE CONTAINERS CAREFULLY.  Damaged containers may leak and in an enclosed space the environment may reach to harmful concentrations of vapours or dust.

If a leaky container is discovered, put on required protective clothing and safety equipment.  After thoroughly airing out the space, enter and take steps to stop the leak.

Cordon off the contaminated area, and post with DANGER AND WARNING SIGNS.

Report the leak to the concerned Sectional Head.

While opening a container or handling toxic or corrosive materials, use required personal protective equipment such as:

Chemical Goggle: For the protection of eyes.

Face Shield: For the protection of face.

P.V.C. Gloves: For the protection of hands.

P.V.C. apron or suit: For the protection of body.

Rubber boot: For the protection of legs.   Suitable respirator, Gas Mask or Breathing apparatus: For the protection of respiratory system.

Empty drums and containers of toxic and corrosive materials should not be used for any purpose unless they have been thoroughly scrubbed and cleaned.

While discarding such empty drums or containers, their sides should be perforated to avoid their use and deposited in a safe area.

Acid Handling

The following precautions should be followed while filling acid in carboy, drums or tankers.

The carboys, drums or tankers should be checked and examined to determine their suitability for storing a particular acid.  These should also be free from any patent defect.

P.V.C. hoses used for transferring acids from fixed pipe line installations to filling point i.e. either carboys, drums or tankers should be checked to see that there are no defects, kinks, bends or leaks in the PVC hoses.

The carboys, drums or tankers should be checked prior to commencement of acid filling operation for likely presence of any organic or foreign material.

The hoses should be laid properly for avoiding any kind or bend.  Any kink or bend in the PVC hose may result in clamps working out loose or even rupturing of PVC hose under pressure.

When not in use, the flexible PVC hoses should be stored in a safe place where they can not be damaged mechanically.

While filling tankers, the vent pipe should be kept open to allow release of pressure and fumes.

The persons who are not connected with filling operations and are only on-lookers should never allowed to remain in close vicinity of acid filling station.

Required protective clothing and safety appliances such as gum boots, PVC suit with hood / PVC apron, acid proof goggles and face shield (when not using hood) must be worn by all persons engaged in any of the acid filling activity viz.  controlling the valve, holding the hose, shifting carboys/drums, closing the carboys/drums, lifting carboys/drums etc.

In case any one is injured with acid splash, his affected part of body should be washed with running water for at least 15 minutes.  For this purpose, safety showers and eye wash fountains are provided at various places.  After washing, medical aid should be obtained.